I thought the "wpa_supplicant" errors were resolved for the Yahboom SD configuration. See item 13 in the FAQ on this page.

G1 Tank-Rpi (yahboom.net)

- Verifying this is the issue

  - Connect to EngTech Wi-Fi (password is Skynet 2023)

  - Open a terminal window and enter the following:

    - ping 8.8.8.8

    - You should see a response. The Pi is connected to the Internet.

    - Now enter the following

    - ping google.com

    - This fails due to error in DNS (Domain Name Server) setting

- Here is the fix

  - Enter the following to make the resolv.conf file writable.

    - sudo chmod +w /etc/resolv.conf

    - If this is successful continue to step b below.

    - If not and you see the error “Operation not permitted”, then do the following:

      - sudo chattr -i /etc/resolve.conf

      - retry the command sudo chmod above. It should now work.

  - Edit the resolv.conf file

    - Enter the following

    - sudo leafpad /etc/resolv.conf

    - You should see this

    - Edit to this, save, and close.

    - Restore to readonly

      - sudo chmod -w /etc/resolv.conf

    - Restore attribute

      - sudo chattr +i /etc/resolve.conf

    - reboot

    - Verify the fix by the following. You should see a response from google.com

      - ping google.com